Hotel Network

Configuration Guide

A Complete Step-by-Step Guide for VLANs, OSPF, DHCP & SSH

3-Floor Hotel Network Topology  |  Cisco Packet Tracer

# Table of Contents

Right-click and select "Update Field" to refresh page numbers

Project Overview	3

Network Topology & Requirements	3

VLAN & IP Addressing Scheme	4

Router Interface Configuration	5

Floor 1 Router	5

Floor 2 Router	5

Floor 3 Router	6

Inter-Router IP Addressing	6

VLAN Configuration on Switches	7

Floor 1 Switch	7

Floor 2 Switch	7

Floor 3 Switch	8

DHCP Configuration	8

Router 1 DHCP Pools	9

Router 2 DHCP Pools	9

OSPF Routing Configuration	10

SSH Remote Access Setup	10

WiFi Access Point Configuration	11

Verification & Testing	11

# Project Overview

This guide walks through the complete configuration of a three-floor hotel network built in Cisco Packet Tracer. The network connects nine departments across three floors using VLAN segmentation, inter-VLAN routing via router-on-a-stick, dynamic IP allocation through DHCP, and OSPF for internal route advertisement.

## Network Topology & Requirements

The hotel infrastructure spans three floors with the following departmental layout:

| Floor | Departments | Devices |
| --- | --- | --- |
| 1st Floor | Reception, Store, Logistics | PCs, Printers, Access Point |
| 2nd Floor | Finance, HR, Sales/Marketing | PCs, Printers, Access Point |
| 3rd Floor | IT, Admin | PCs, Printers, Access Point |

Table 1: Floor-by-Floor Department Layout

Each department operates on its own VLAN with dedicated subnet addressing. All departments require wireless connectivity via access points, printer and PC access, and full inter-departmental communication.

## VLAN & IP Addressing Scheme

The network uses a consistent addressing pattern across all departments. Each floor's router serves as the DHCP server for its local VLANs. The table below summarizes the complete IP plan:

| Floor | Department | VLAN | Network | Gateway |
| --- | --- | --- | --- | --- |
| Floor 1 | Reception | 80 | 192.168.8.0/24 | 192.168.8.1 |
|  | Store | 70 | 192.168.7.0/24 | 192.168.7.1 |
|  | Logistics | 60 | 192.168.6.0/24 | 192.168.6.1 |
| Floor 2 | Finance | 50 | 192.168.5.0/24 | 192.168.5.1 |
|  | HR | 40 | 192.168.4.0/24 | 192.168.4.1 |
|  | Sales | 30 | 192.168.3.0/24 | 192.168.3.1 |
| Floor 3 | Admin | 20 | 192.168.2.0/24 | 192.168.2.1 |
|  | IT | 10 | 192.168.1.0/24 | 192.168.1.1 |

Table 2: VLAN and IP Addressing Scheme

## Inter-Router Links

The three routers connect to each other via serial DCE cables using /30 subnets to minimize address waste on point-to-point links:

| Link | Subnet | Router IPs |
| --- | --- | --- |
| Router 1 - Router 2 | 10.10.10.0/30 | R1: .1, R2: .2 |
| Router 1 - Router 3 | 10.10.10.4/30 | R1: .5, R3: .6 |
| Router 2 - Router 3 | 10.10.10.8/30 | R2: .10, R3: .9 |

Table 3: Router-to-Router Link Addresses

# Router Interface Configuration

Before assigning IP addresses or configuring routing, all router interfaces must be activated. This section covers the physical interface bring-up process for each floor's router.

## Floor 1 Router

Activate the serial interfaces connecting to Floor 2 and Floor 3 routers, plus the GigabitEthernet interface facing the local switch:

Router> enable

Router# configure terminal

Router(config)# interface serial0/2/0

Router(config-if)# no shutdown

Router(config-if)# interface serial0/2/1

Router(config-if)# no shutdown

Router(config-if)# interface gigabitEthernet0/0

Router(config-if)# no shutdown

Router(config-if)# do write

## Floor 2 Router

Floor 2 router connects to both Floor 1 and Floor 3 via serial links. Remember to set the clock rate on the DCE end:

Router> enable

Router# configure terminal

Router(config)# interface serial0/1/1

Router(config-if)# no shutdown

Router(config-if)# interface serial0/1/0

Router(config-if)# no shutdown

Router(config-if)# interface gigabitEthernet0/0

Router(config-if)# no shutdown

Router(config-if)# do write

! Set clock rate on DCE interface

Router(config)# interface serial0/1/1

Router(config-if)# clock rate 64000

Router(config-if)# do write

## Floor 3 Router

Activate Floor 3 router's interfaces and configure the clock rate:

Router> enable

Router# configure terminal

Router(config)# interface serial0/2/0

Router(config-if)# no shutdown

Router(config-if)# interface serial0/2/1

Router(config-if)# no shutdown

Router(config-if)# interface gigabitEthernet0/0

Router(config-if)# no shutdown

Router(config-if)# do write

! Set clock rate on DCE interface

Router(config)# interface serial0/2/0

Router(config-if)# clock rate 64000

Router(config-if)# do write

# Inter-Router IP Addressing

After bringing interfaces up, assign IP addresses to the serial links between routers. These /30 subnets provide exactly two usable host addresses per link.

## Router 1 (Floor 1)

Router> enable

Router# configure terminal

Router(config)# interface serial0/2/0

Router(config-if)# ip address 10.10.10.5 255.255.255.252

Router(config-if)# interface serial0/2/1

Router(config-if)# ip address 10.10.10.9 255.255.255.252

Router(config-if)# do write

## Router 2 (Floor 2)

Router> enable

Router# configure terminal

Router(config)# interface serial0/1/0

Router(config-if)# ip address 10.10.10.1 255.255.255.252

Router(config-if)# interface serial0/1/1

Router(config-if)# ip address 10.10.10.10 255.255.255.252

Router(config-if)# do write

## Router 3 (Floor 3)

Router> enable

Router# configure terminal

Router(config)# interface serial0/2/0

Router(config-if)# ip address 10.10.10.6 255.255.255.252

Router(config-if)# interface serial0/2/1

Router(config-if)# ip address 10.10.10.2 255.255.255.252

Router(config-if)# do write

# VLAN Configuration on Switches

Each floor has a dedicated switch where VLANs are created and ports are assigned. The trunk port (FastEthernet0/1) connects to the floor's router for router-on-a-stick operation. Access ports connect to departmental PCs, printers, and access points.

## Floor 1 Switch

Configure VLANs 80 (Reception), 70 (Store), and 60 (Logistics). Set FastEthernet0/1 as trunk:

Switch> enable

Switch# configure terminal

! Reception VLAN 80

Switch(config)# interface range fa0/2-3

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 80

! Store VLAN 70

Switch(config)# interface range fa0/6-8

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 70

! Logistics VLAN 60

Switch(config)# interface range fa0/9-11

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 60

! Trunk to Router

Switch(config)# interface range fa0/1

Switch(config-if-range)# switchport mode trunk

Switch(config-if-range)# do write

## Floor 2 Switch

Configure VLANs 50 (Finance), 40 (HR), and 30 (Sales/Marketing):

Switch> enable

Switch# configure terminal

! Finance VLAN 50

Switch(config)# interface range fa0/2-3

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 50

! HR VLAN 40

Switch(config)# interface range fa0/4-5

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 40

! Sales VLAN 30

Switch(config)# interface range fa0/6-8

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 30

! Trunk to Router

Switch(config)# interface range fa0/1

Switch(config-if-range)# switchport mode trunk

Switch(config-if-range)# do write

## Floor 3 Switch

Configure VLANs 10 (IT) and 20 (Admin):

Switch> enable

Switch# configure terminal

! IT VLAN 10

Switch(config)# interface range fa0/2-3

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 10

! Admin VLAN 20

Switch(config)# interface range fa0/4-6

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 20

! Trunk to Router

Switch(config)# interface range fa0/1

Switch(config-if-range)# switchport mode trunk

Switch(config-if-range)# do write

Note: VLANs are automatically created when first referenced via the switchport access vlan command. The trunk port carries all VLAN traffic to the router for inter-VLAN routing.

# DHCP Configuration

Each router acts as the DHCP server for its floor's VLANs using router-on-a-stick. Sub-interfaces are created on the GigabitEthernet0/0 interface, each tagged with its VLAN and assigned the gateway IP for that subnet.

## Router 1 DHCP Pools (Floor 1)

Create sub-interfaces and DHCP pools for Reception, Store, and Logistics:

Router> enable

Router# configure terminal

! Sub-interface for Reception (VLAN 80)

Router(config)# interface gigabitEthernet0/0.80

Router(config-subif)# encapsulation dot1Q 80

Router(config-subif)# ip address 192.168.8.1 255.255.255.0

! Sub-interface for Store (VLAN 70)

Router(config)# interface gigabitEthernet0/0.70

Router(config-subif)# encapsulation dot1Q 70

Router(config-subif)# ip address 192.168.7.1 255.255.255.0

! Sub-interface for Logistics (VLAN 60)

Router(config)# interface gigabitEthernet0/0.60

Router(config-subif)# encapsulation dot1Q 60

Router(config-subif)# ip address 192.168.6.1 255.255.255.0

! Enable DHCP service

Router(config)# service dhcp

! DHCP Pool: Reception

Router(config)# ip dhcp pool Reception

Router(dhcp-config)# network 192.168.8.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.8.1

Router(dhcp-config)# dns-server 192.168.8.1

Router(dhcp-config)# exit

! DHCP Pool: Store

Router(config)# ip dhcp pool Store

Router(dhcp-config)# network 192.168.7.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.7.1

Router(dhcp-config)# dns-server 192.168.7.1

Router(dhcp-config)# exit

! DHCP Pool: Logistics

Router(config)# ip dhcp pool Logistics

Router(dhcp-config)# network 192.168.6.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.6.1

Router(dhcp-config)# dns-server 192.168.6.1

Router(dhcp-config)# do write

## Router 2 DHCP Pools (Floor 2)

Create sub-interfaces and DHCP pools for Finance, HR, and Sales:

Router> enable

Router# configure terminal

! Sub-interface for Sales (VLAN 30)

Router(config)# interface gigabitEthernet0/0.30

Router(config-subif)# encapsulation dot1Q 30

Router(config-subif)# ip address 192.168.3.1 255.255.255.0

! Sub-interface for HR (VLAN 40)

Router(config)# interface gigabitEthernet0/0.40

Router(config-subif)# encapsulation dot1Q 40

Router(config-subif)# ip address 192.168.4.1 255.255.255.0

! Sub-interface for Finance (VLAN 50)

Router(config)# interface gigabitEthernet0/0.50

Router(config-subif)# encapsulation dot1Q 50

Router(config-subif)# ip address 192.168.5.1 255.255.255.0

! Enable DHCP service

Router(config)# service dhcp

! DHCP Pool: Finance

Router(config)# ip dhcp pool Finance

Router(dhcp-config)# network 192.168.5.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.5.1

Router(dhcp-config)# dns-server 192.168.5.1

Router(dhcp-config)# exit

! DHCP Pool: HR

Router(config)# ip dhcp pool HR

Router(dhcp-config)# network 192.168.4.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.4.1

Router(dhcp-config)# dns-server 192.168.4.1

Router(dhcp-config)# exit

! DHCP Pool: Sales

Router(config)# ip dhcp pool Sales

Router(dhcp-config)# network 192.168.3.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.3.1

Router(dhcp-config)# dns-server 192.168.3.1

Router(dhcp-config)# do write

Tip: After configuring DHCP, go to any end device (PC, laptop), open Desktop > IP Configuration, and select DHCP. The device will automatically receive an IP address from the appropriate pool.

# OSPF Routing Configuration

OSPF (Open Shortest Path First) is used to dynamically advertise routes between all three routers. All networks are placed in Area 0 (backbone area) for this single-area design.

## Router 1 OSPF

Router> enable

Router# configure terminal

Router(config)# router ospf 10

Router(config-router)# network 10.10.10.4 0.0.0.3 area 0

Router(config-router)# network 10.10.10.8 0.0.0.3 area 0

Router(config-router)# network 192.168.8.0 0.0.0.255 area 0

Router(config-router)# network 192.168.7.0 0.0.0.255 area 0

Router(config-router)# network 192.168.6.0 0.0.0.255 area 0

Router(config-router)# do write

## Router 2 OSPF

Router> enable

Router# configure terminal

Router(config)# router ospf 10

Router(config-router)# network 10.10.10.0 0.0.0.3 area 0

Router(config-router)# network 10.10.10.8 0.0.0.3 area 0

Router(config-router)# network 192.168.5.0 0.0.0.255 area 0

Router(config-router)# network 192.168.4.0 0.0.0.255 area 0

Router(config-router)# network 192.168.3.0 0.0.0.255 area 0

Router(config-router)# do write

## Router 3 OSPF

Router> enable

Router# configure terminal

Router(config)# router ospf 10

Router(config-router)# network 10.10.10.0 0.0.0.3 area 0

Router(config-router)# network 10.10.10.4 0.0.0.3 area 0

Router(config-router)# network 192.168.2.0 0.0.0.255 area 0

Router(config-router)# network 192.168.1.0 0.0.0.255 area 0

Router(config-router)# do write

Note: OSPF uses wildcard masks (inverse of subnet masks). For a /24 network, the wildcard is 0.0.0.255; for a /30, it is 0.0.0.3.

# SSH Remote Access Setup

SSH (Secure Shell) provides encrypted remote management access to the routers. Configure SSH on all three routers using the following procedure:

1. Set hostname and domain name:

2. Generate RSA key pair:

3. Create local user account:

4. Enable SSH and configure VTY lines:

Repeat these steps on all three routers, substituting appropriate hostnames (Floor2-Router, Floor3-Router).

# WiFi Access Point Configuration

Each floor has wireless access points connected to the switch, allowing mobile devices to connect wirelessly to their respective VLAN networks.

To configure an access point:

• Connect the access point to a switch port configured in the appropriate VLAN

• Open the access point configuration in Packet Tracer

• Set the SSID (e.g., Hotel-Floor1, Hotel-Floor2, Hotel-Floor3)

• Configure WPA2-PSK authentication with a strong passphrase

• Ensure the port is in access mode for the desired VLAN

Wireless devices (laptops, smartphones) will receive IP addresses via DHCP from the same pools as wired devices on the same VLAN.

# Verification & Testing

After completing all configurations, verify connectivity using these commands and tests:

## Quick Verification Checklist

| Check | Command |
| --- | --- |
| Interface status | show ip interface brief |
| VLAN configuration | show vlan brief |
| OSPF neighbors | show ip ospf neighbor |
| Routing table | show ip route |
| DHCP leases | show ip dhcp binding |
| SSH status | show ip ssh |

Table 4: Verification Commands

## End-to-End Testing

From any PC on any VLAN, open the Command Prompt and test connectivity to devices on other floors:

C:\> ping 192.168.1.10     ! Ping IT department from Reception

C:\> ping 192.168.5.10     ! Ping Finance from Logistics

C:\> ping 192.168.3.10     ! Ping Sales from Admin

C:\> tracert 192.168.2.10  ! Trace route to Admin

Successful pings confirm that VLAN routing, OSPF neighbor relationships, and inter-floor connectivity are all working correctly.

Hotel Network

Configuration Guide

Cisco Packet Tracer  |  Networking Fundamentals

VLANs  |  OSPF  |  DHCP  |  SSH  |  Router-on-a-Stick